Group Members:

MUHAMMAD MOIN DILDAR		(211201) MUHAMMAD MAHMOOD GHAURI		(211265) MUHAMMAD SALMAN	(211291)

BE MECHATRONICS (Session 2021-2025)

Project Supervisor Engr. Umer Farooq

Lecturer

DEPARTMENT OF MECHATRONICS & BIOMEDICAL ENGINEERING FACULTY OF ENGINEERING

AIR UNIVERSITY, ISLAMABAD

i

FINAL YEAR PROJECT REPORT (SESSION 2021-2025)

DEPARTMENT OF MECHATRONICS & BIOMEDICAL ENGINEERING

Submitted By:

MUHAMMAD MOIN DILDAR		(211201) MUHAMMAD MAHMOOD GHAURI			(211265) MUHAMMAD SALMAN	(211291)

Project Supervisor

Engr. Umer Farooq

Lecturer

Chair Department

Dr. Noman Naseer

Acknowledgments

### The successful completion of this project would not have been possible without the invaluable support and contributions of several individuals and entities.

### We would like to express our sincere gratitude to Engr. Umer Farooq, our project supervisor, Dr. Hafiz Zia Ur Rehman, our project co-supervisor and all other faculty members for their continuous guidance, encouragement, and insightful feedback throughout the development process. Their expertise and support were instrumental in shaping this project.

### We are also thankful to our team members, for their assistance and their contributions were crucial in this project. Furthermore, we would like to acknowledge DEPARTMENT OF MECHATRONICS & BIOMEDICAL ENGINEERING, for providing the resources and necessary support to carry out this project. This support played a vital role in ensuring the project's success.

Abstract

Nonintrusive load monitoring (NILM) systems serve to uncover the energy usage patterns of individual devices within an electrical system, yet expanding their market reach poses a significant hurdle. A novel approach to NILM, utilizing edge processing, is introduced, wherein energy consumption data undergo processing directly on the device installed within the monitored facility. Specifically, a neural network implemented on a Raspberry pi 4B (8GB RAM) is employed for this purpose. The NILM system is slated for installation on four everyday using appliances in real-world scenarios. This report outlines a promising configuration aimed at facilitating the widespread adoption of NILM frameworks by reducing their implementation costs and complexity, while also addressing privacy concerns associated with cloud-based data processing. The outcomes of real-world testing in this endeavor are expected to provide compelling evidence of the potential of the proposed NILM framework across various applications, including smart homes, building automation, and industrial energy management.

Nomenclature

#### Symbol	Abbreviation

| A | Unit of Current |
| --- | --- |
| AI | Artificial Intelligence |
| ANN | Artificial Neural Network |
| CNN | Convolutional Neural Network |
| DL | Deep Learning |
| FHMM | Factorial Hidden Markov Model |
| flin | Linear Frequency |
| HMMs | Hidden Markov Models |
| Hz | Unit of Frequency |
| ILM | Intrusive Load Monitoring |
| Imax | Maximum Current |
| Inom | Normal Current |
| NILM | Non-Intrusive Load Monitoring |
| NN | Neural Network |
| RMS | Root Mean Square |
| V | Unit of Voltage |
| Vnom | Normal Voltage |
| Vmax | Maximum Voltage |

1.1	Background and Motivation	2

1.1.1	Sustainable Development Goals	2

1.2	Literature Review	2

1.3	Problem Statement	5

1.4	Objectives of the Project	5

1.5	Cost Analysis	6

1.6	Timeline of the Project	6

1.7	Work Division	6

1.8	Organization of Report	7

2.1	Description of the Project	8

2.2	Methodology	9

2.3	Product Specifications	11

2.3.1	Schematic Diagram	12

2.4	Preliminary Design and Calculations	12

3.1	Algorithms	13

3.1.1	Flowcharts	14

4.1	Hardware Description	16

4.2	Components	17

5.1	Results	19

5.2	Discussion	21

5.3	Project Outcome	22

5.4	Project Impact	22

6.1	Conclusion	23

6.2	Future Recommendation	23

Figure 1: Timeline of Project	7

Figure 2: Code Flow inform of Flow Chart	9

Figure 3: Block Diagram of NILM system	10

Figure 4: Block Diagram of System during Data Set collection phase	10

Figure 5: Schematic Diagram of System	12

Figure 6: Preliminary Design of Final Product	12

Figure 7: Flow of Code in SEQ2Point	15

Figure 8: Hardware setup Installed for Data-Set Collection	16

Figure 9: Features of Dataset	16

List of Tables

Table 1: Cost Estimation	6

Table 2: List of Hardware Components	17

    - Introduction

Energy management is a critical aspect of modern society, with increasing demands for efficient utilization of resources and sustainable practices. In response to this need, this project focuses on developing an AI-based energy monitoring embedded system. This system integrates advanced technologies such as Non-Intrusive Load Monitoring (NILM), Neural Networks (NN), and hardware components like the Raspberry Pi 4B. Today, it's really important for businesses and homes to keep an eye on how much energy they use. This project is all about helping with that. This project tends to create a smart system that can track energy use in real- time. In a world where everything costs money and resources are precious, it's really useful to know where the energy is consumed. This project isn't just about looking at numbers; it's about helping user to make better choices about how they use energy. With this system, user can see exactly where energy is being used, find ways to use less, and save money on energy bills. When humans use energy more carefully, pollution can be reduced and make the world a cleaner, healthier place for everyone.

# Background and Motivation

## Sustainable Development Goals

### Today’s world is using more energy than ever, and it's hurting the environment. That's why we need smarter ways to track how much energy we're using.

### The old way of monitoring energy was Intrusive Load Monitoring (ILM) which needs each monitoring device to be installed on every appliance, which could be challenging sometimes to be implemented practically. This project tackles that problem with an approach called Non-Intrusive Load Monitoring (NILM). In which a system can analyze overall energy usage and figure out how much each appliance is using by analyzing the input AC voltage signal, all without installing each monitoring device on each appliance.

### NILM uses artificial intelligence (AI) to become a super-powered detective for energy usage. It can analyze energy usage in real-time, helping user understand where user can save money and be kinder to the environment. This project dives into the world of AI- powered NILM, exploring its potential to revolutionize how we manage energy

# Literature Review

### Since Hart [1] introduced the primary NILM framework during the 1980s, there have been huge progressions in the field. All through the accompanying twenty years, research basically centered around finding novel marks prepared to do precisely recognizing gadgets and creating classifiers to decipher these marks. This approach frequently involves recognizing occasions prior to classifying them. When an occasion is distinguished, the related machine's elements (and accordingly its mark) are separated. This technique can be broken down into three key stages: occasion recognition, include extraction, and burden ID. These procedures are normally sorted inside an occasion-based structure, where an "occasion" alludes to any adjustment of the electrical boundaries of the total sign.

### Dong et al. [2] proposed a framework focused on distinguishing power signal occasions and their connected boundaries, including dynamic power range, receptive power range, consonant substance range, presence of spikes, number of stages, and occasion search time. The recognized occasions are then connected with machine functional cycles utilizing a bunching calculation. In [3], a comparative methodology is taken, connecting identified power signal occasions to machines by limiting contrasts across different boundaries, for example, successful voltage, compelling current, dynamic power, receptive power, obvious power, power factor, complete symphonious bending, and voltage-current direction. Over the long haul, endeavors have been pointed toward recognizing highlights extractable from occasions that empower exact machine separation while preferably staying predictable across various working circumstances. Thus, Teshome et al [4] disintegrated the total current into two symmetrical parts and characterized V-I directions comparative with every part.

### Gupta et al. [5], further investigated this road by ceaselessly observing the electromagnetic obstruction impacts created by machines during their initiation or deactivation. Their framework processes the voltage estimated at a family attachment progressively to get its Fourier change. Post-handling uncovers a striking expansion in symphonic substance following machine exchanging occasions. Recognizable proof of the machine liable for the noticed symphonic substance is accomplished through a k-closest neighbor classifier. Essentially, in [6], the effect of turning procedure on the retained current sign was surveyed to create galvanically detached estimation frameworks. Be that as it may, these strategies share a few limits: Right off the bat, the adequacy of occasion location calculations is tested by the trouble in adjusting bogus up-sides and misleading negatives. Commotion in total power flags frequently clouds the distinguishing proof of minor burdens. While recently proposed occasion-based frameworks show remarkable execution because of precisely quantifiable post-occasion highlights, for example, those referenced prior, they display unfortunate speculation abilities, particularly when sent in concealed families or when prepared on information from different homes [7]. Furthermore, a considerable lot of these frameworks face computational difficulties that raise essentially with the quantity of burdens to be dis-collected, delivering them unrealistic for true applications [4].

### Lastly, while many of these systems can detect appliance activities, they often lack the ability to provide quantitative insights into energy consumption. Non-intrusive acquisition of information regarding the status of various appliances holds significance in numerous

### applications where NILM systems find utility, including smart home automation and ambient assisted living. NILM systems serve to delineate the energy consumption patterns of individual appliances, potentially facilitating the development of recommendation systems.

### Around 2010, a prominent shift arose in the domain of NILM frameworks research: the ascent of frameworks that sidestep the underlying occasion discovery stage. These frameworks work on a ceaseless stream of total sign examples (basically time-series information), handling them without hanging tight for explicit occasions. Named non- occasion-based frameworks, they take out the requirement for elements or marks, depending rather on the total power signal itself. Kolter et al. [8] were among the trailblazers in this field, presenting frameworks in light of discriminative meager coding for energy disaggregation. This technique includes preparing discriminative models for various machine classes, with individual energy utilization determined as a mix of essential capabilities increased by initiations.

### Secret Markov models (Gee) have turned into a staple in non-occasion based NILM frameworks. Kim et al. [9] were early defenders, presenting a factorial Well (FHMM) where every machine's way of behaving is displayed freely. Following a preparation stage, the FHMM can gather machine states from the total utilization signal, empowering individual utilization division. In [10], Well are coordinated into a Bayesian structure, amalgamating different machine models into an exhaustive machine model. Bonfigli et al.

### [11] proposed a bivariate FHMM utilizing both active and reactive power consumption data. Additionally, Paradiso et al. [12] exhibited that consolidating advantageous data like house inhabitance and machine use times can upgrade disaggregation results.

### In 2015, Kelly and Knottenbelt [13] in presented the utilization of profound learning (DL) in non-occasion based NILM frameworks. While fake brain organizations (ANNs) had been used beforehand in NILM research, they were ordinarily utilized as classifiers in occasion-based frameworks' heap recognizable proof stages. [13] for the initial time, the total power signal was handled through an ANN utilizing moving-window handling, regarding the issue as a visually impaired source partition task. The creators exhibited that such designs beat combinatorial streamlining [8] and FHMM models [9], [10]. DL algorithms offer the benefit of computerizing highlight extraction, learning undertakings, for example, individual apparatus utilization, on-off state advances, and functional lengths straightforwardly from the information utilizing ANNs.

### [14] Sense tracks your home's energy, it starts to perceive most apparatuses and different gadgets that utilization in excess of 60 watts. Normally the Sense screen identifies 12 gadgets in the

### principal month after it's introduced and 25-30 gadgets following a year. Each house is unique, notwithstanding, so the quantity of recognized gadgets might be sequential. Sense is intended for fast establishment and consistent improvement. The screen utilizes sensors that read the electrical flow north of 1 million times each second, while AI utilizes that information to figure out what gadgets are here and there.

### [15] In this study, a Nonintrusive Load Monitoring (NILM) system leveraging edge processing is introduced, meaning to observe the energy utilization ways of behaving of individual gadgets inside an electrical arrangement. The proposed framework conducts energy utilization information handling straightforwardly on the gadget introduced inside the observed office. In particular, it embraces a succession to-point approach utilizing a convolutional brain network executed on an Arm CortexM7 microcontroller. Moreover, the paper presents discoveries from a broad year testing stage. The NILM framework was conveyed in two genuine homes in focal Italy to survey it’s down to earth sending and likely utility in true settings. This exploration offers a promising arrangement ready to work with the far-reaching reception of NILM frameworks, tending to worries in regards to execution expenses, intricacy, and protection related with cloud- based information handling. The results of true testing highlight the proposed NILM framework's possible across different spaces, including brilliant homes, building computerization, and modern energy the board.

# Problem Statement

The current landscape of energy consumption monitoring lacks efficient real-time capabilities, often relying on intrusive hardware installations but that could be challenging to practically implement. To address this, there is a critical need for a non- intrusive load management system (NILM) powered by artificial intelligence (AI) that can accurately monitor energy consumption in real-time, including individual appliance energy usage.

# Objectives of the Project

### Identification of Energy consumption of individual device from the selected group of daily use household appliances.

### Real Time Data collection (stored on server) and by analyzing that data to predict the possible power consumption by a specific appliance.

### Implementing AI algorithm in Microcontroller.

# Cost Analysis

Table 1: Cost Estimation

| Item | No. of Items | Price |
| --- | --- | --- |
| Raspberry pi 4B | 1 | 25,500 |
| ESP 32 | 3 | 3,400 |
| Sensors | 6 | 15,300 |
| SD Card Module | 2 | 300 |
| RTC DS3231 | 2 | 1,300 |
| Wires & Sockets |  | 7,700 |
| Display, SD Cards & Other miscellaneous items |  | 7,500 |
| Total |  | 61,000 |

# Timeline of the Project

Figure 1: Timeline of Project

# Work Division

### Research work, theoretical background and AI algorithm training & Implementation will be performed by MUHAMMAD SALMAN.

### PCB designing,	hardware	design and	implementation Code implementation (for data set collection) will be performed by MUHAMMAD MAHMOOD GHAURI.

### Deployment of trained AI algorithm will be performed by MUHAMMAD MOIN DILDAR.

# Organization of Report

### In Chapter 1, an introduction is presented and discusses the background motivation to perform this project. Also mentioned the previous work done on technologies used in this project. Chapter 1 discusses why today’s world needs it and what objectives are to be achieved. Also, what cost and timeline required to do this project.

### Chapter 2 discusses the final product specifications and approach used to achieve the above- mentioned objectives.

    - Description

Chapter 1, the background, motivations, and already research related to AI-powered energy management systems had been studied, In Chapter 2, further searched on the project. Here, a detailed description of the system itself, outlining the methodology behind its development, product specifications, and a preliminary schematic diagram. And also delve into the design considerations and initial calculations that will guide the project's implementation.

# Description of the Project

### This project aims to monitor energy consumption using smart technology called Non- Intrusive Load Monitoring (NILM) Using Artificial Intelligence (AI). The main goal of this device is to help businesses and households to keep track of their energy usage in a simple and efficient way. The project consists of several components, including the design of the monitoring system, the integration of NILM and AI technologies using Raspberry Pi 4B (4GB RAM), ESP 32, sensors (Smart Samaan single phase energy metering module) and Display. The device works by analyzing overall energy consumption data to identify patterns associated with specific appliances or behaviors, without the need for intrusive methods like installing sensors on individual devices. It helps businesses and homes to save money by finding ways to reduce their energy usage and to protect the environment by promoting energy-saving practices that reduce pollution and save resources and it makes energy monitoring easier for everyone, so more people can manage their energy usage better.

# Methodology

Figure 2: Code Flow inform of Flow Chart

Figure 3: Block Diagram of NILM system

Figure 4: Block Diagram of System during Data Set collection phase

### In Figure 2, flow of code is represented in the form of flowchart. The system will extract feature and AI algorithm by using these features detects which appliance is consuming energy and the results will be shown on display. Also results will be transmitted to wireless monitoring station.

### In Figure 4, the system is shown in block diagram form during data set generation where the system will act as an intrusive load monitoring system in which signal from each appliance will be stored in a storage device and overall signal of combined load of input will be analyzed and stored.

### In Figure 3, the final product is shown on block diagram level in which system will be Non- Intrusive Load Monitoring system. The system will only analyze input signals and through Energy Disaggregation technique, energy consuming device and energy consumption by each device will be detected.

# Product Specifications

### Here, the list of features and operational specifications of our project is mentioned.

### Real-Time Monitoring: The device provides up-to-date information on energy consumption, permitting users to path usage patterns as they occur.

### Non-Intrusive Operation: Using Non-Intrusive Load Monitoring (NILM) technology, the system analyzes overall power consumption without the need for intrusive hardware installation.

### AI Integration: By including Artificial Intelligence, the device can identify individual appliance usage patterns and provide insights for monitor and display.

### User-Friendly Interface: The device features a simple interface that permits users to easily access and illuminate energy usage data.

### Environmental Sustainability: It helps protect the environment by promoting energy- saving practices that reduce pollution and save resources and it makes energy monitoring easier for everyone, so more people can manage their energy use better

## Schematic Diagram

Figure 5: Schematic Diagram of System

During Data set collection phase ILM System will be implemented and In final product NILM system will be implemented.

# Preliminary Design and Calculations

- Vmax = 840 Vpeak

- Imax = 43 A

Figure 6: Preliminary Design of Final Product

    - Modeling & Simulation

# Algorithms

Algorithm for Sequence-to-Point NILM

### This algorithm describes the steps to implement the Sequence-to-Point CNN model for NILM, from data preprocessing to real-time deployment.

- Input Aggregate Power Data

### Obtain aggregate power measurements from a smart meter or an energy monitoring system.

- Preprocess the Data

### Normalization: Adjust the power data to achieve a mean of 0 and a standard deviation of 1.

### Sliding Window Method: Split the data into fixed-size overlapping segments (for example, 599 samples per segment).

### Designate the power reading of the appliance at the center of each segment as the target value for training.

- Configure the Sequence-to-Point CNN Model

### Specify a CNN architecture that includes convolutional layers for feature extraction and fully connected layers for output regression.

### Input: A sequence of aggregate power samples (e.g., 599 samples).

### Output: A single value indicating the power consumption of the appliance at the midpoint.

    - Train the Model

### Utilize the prepared dataset to train the CNN model:

### Loss Function: Aim to minimize the Mean Squared Error (MSE) between predicted and actual outcomes

### Optimizer: Implement the Adam optimizer to achieve quicker convergence

### Conduct training in batches and perform multiple epochs of iteration.

- Validate and Test the Model

### Evaluate the trained model on validation and test datasets using:

    - Mean Absolute Error (MAE): Average error in predictions.

    - Signal Aggregate Error (SAE): Measure of total energy prediction accuracy.

- Quantize the Model for Deployment

### a. Convert the model weights from 32-bit floating-point to 8-bit integers to reduce size and computational requirements.

- Deploy the Model on an Embedded Device

### a. Deploy the quantized model on a microcontroller (e.g., NUCLEO-H743ZI2) or other embedded systems.

- Real-Time Processing

### Feed real-time aggregate power readings to the model.

### Use the sequence-to-point structure to predict appliance-level power consumption for the current window.

- Output Appliance-Level Power

### a. Generate real-time predictions of power usage for individual appliances, enabling energy monitoring and analysis.

## Flowcharts

### The flowchart shows the steps for using a sequence-to-point CNN model for Non- Intrusive Load Monitoring (NILM). It starts with collecting total power usage data, which is processed by normalizing and splitting it into smaller windows. A CNN model is trained to predict the power usage of individual appliances from this data. After training, the model is simplified (quantized) to run on a small device like a microcontroller. Once deployed, it processes live power data and provides real- time predictions of how much power each appliance is using.

Figure 7: Flow of Code in SEQ2Point

- Experimental Setup

Figure 8: Hardware setup Installed for Data-Set Collection

# Hardware Description

For data set collection ILM system is implemented in which each sensor is reading the features of each appliance. Also, one sensor is reading features of input AC supply. These sensors are serially transmitting features to ESP32s and then ESP32s are storing these features in SD card & cloud-based Firebase database. Real time clock module to get unix timestamp for dataset.

Features which we will be extracting during Dataset collection are shown in following figure:

Figure 9: Features of Dataset

# Components

Table 2: List of Hardware Components

| Single Phase Energy Metering Module by Smart Samaan |  |
| --- | --- |
| ESP32 |  |
| RTC DS3231 |  |
| SD Card Module |  |
| OLED Display |  |
| 5V, 2A DC Supply Charger |  |
| Wires (3/29, 7/29) |  |
| Raspberry Pi 4B (8GB RAM) |  |
| TFT Display |  |

- Results and Discussion

The Seq2Point show effectively forecasted four domestic appliances dynamic control consumption Iron, Vacuum, Fridge, and Oven and their ON/OFF states classification. The relapse errand shown a moo cruel supreme mistake (MAE), reflecting exact control estimation, and the classification assignment displayed a tall F1 score, reflecting reliable location of gadget operation cycles. The convolutional structure of the demonstrate backed significant highlight extraction from time-series input information, upgrading generally forecast precision. Comparisons of visual genuine and anticipated control values guaranteed the solidness of the Seq2Point show. Trading the prepared show to TensorFlow Lite made it conceivable to convey effectively on Raspberry Pi 4, which is perfect for real-time vitality observing.

# Results

In this Project, we built and prepared a Seq2Point demonstrate to foresee mutually the dynamic control utilization and operational state (ON/OFF) of four domestic machines: Press, Vacuum, Ice chest, and Stove. The show might learn from the input highlights and make rectify estimations for both relapse and classification tasks.

At test time, the show delivered an amazingly moo Cruel Supreme Blunder (MAE) of 0.00518 for anticipating control, reflecting tall exactness in approximating vitality utilization. The Normalized Disaggregation Mistake (NDE) was 0.2323, once more affirming to the model's capacity to closely track genuine control signals. Within the classification assignment, the demonstrate achieved an amazing F1 Score of 0.9962, reflecting sublime precision in recognizing apparatus ON/OFF status.

We too tried the execution of the show utilizing visual comparisons of genuine and evaluated control values, which confirmed tight assention between real and evaluated information. The comes about hence appear that the show generalizes to inconspicuous information and can be trusted in commonsense use.

In expansion, the show was effectively ported to the TensorFlow Lite organize and executed on a Raspberry Pi 4 board. This illustrates the model's status for inserted, real-time vitality observing frameworks utilized in savvy homes.

Overall, the extend shows promising signs of improving vitality proficiency and keen gadget administration through correct disaggregation of control utilization in a multi-device setup.

Figure 10: Trained Model Predicted Output of Iron

Figure 11: Trained Model Predicted Output of Vacuum

Figure 12: Trained Model Predicted Output of Fridge

Figure 13: Trained Model Predicted Output of Oven

Figure 14: Hardware of Final Meter implementing NILM System

# Discussion

The extend was completed independently in different parts with the point of accomplishing generally adjust vitality disaggregation. The dataset was to begin with accumulated from family apparatuses with both control utilization values and operational cycle statuses. The crude information was at that point preprocessed, normalized, and organized for demonstrate preparing with most extreme care.

Subsequently, windowed arrangements of input information were built utilizing a sliding window approach to account for transient connections within the control signals. These arrangements contained normalized timestamps and add up to dynamic control as the inputs to the model.

A Seq2Point multitask profound learning show was created and prepared to perform two errands at once: relapse for determining dynamic control utilization of four gadgets, and classification to recognize their ON/OFF states. Strategies counting dropout layers and callbacks such as early halting and learning rate lessening were utilized to optimize preparing and avoid overfitting.

The demonstrate was tried on another free test set and gave amazing execution measurements a Cruel Supreme Blunder (MAE) of 0.00518 and Normalized Disaggregation Blunder (NDE) of 0.2323 for control forecast, and a fabulous F1 Score of 0.9962 for gadget state classification.

Visual comparison of real versus evaluated control utilization advance outlined the model's tall

exactness. In conclusion, the demonstrate was optimized to TensorFlow Lite and effectively actualized on a Raspberry Pi 4, highlighting its real-world utilization on implanted hardware.

Overall, the project included information collection, preprocessing, multitask learning, exhaustive assessment, and real-world arrangement â effectively giving a effective and effective arrangement for appliance-level vitality checking.

# Project Outcome

- Successfully collected and prepared a comprehensive dataset containing power consumption and operational cycle data for multiple household appliances.

- Developed a multitask Seq2Point deep learning model that performs both regression (predicting active power) and classification (detecting ON/OFF states) tasks simultaneously.

- Achieved high performance metrics with a Mean Absolute Error (MAE) of 0.00518 for power prediction and an F1 Score of 0.9962 for classification accuracy.

- Implemented effective training techniques including dropout layers and callbacks to prevent overfitting and optimize learning.

- Validated the model’s predictions with visual plots comparing true and predicted values, confirming strong alignment.

- Converted the trained model to TensorFlow Lite format and deployed it on a Raspberry Pi 4, demonstrating practical real-time application capability.

- Provided a scalable and cost-effective solution for detailed energy monitoring in residential settings.

# Project Impact

This project represents a major step forward in home energy monitoring by allowing for accurate, device-level power usage analysis. The multitask Seq2Point model's capability of predicting power consumption and device status jointly provides an efficient and effective solution that surpasses conventional single-task approaches.

By running the model on a low-cost Raspberry Pi 4, the project closes the loop between high-level AI methods and low-cost practical hardware implementation. This renders intelligent energy management more accessible to a wider audience, promoting energy-saving habits.

The precise monitoring capability can help reduce wasteful power consumption, lower electricity bills, and promote environmental sustainability through minimizing carbon footprints.

In addition, the scalability of this method makes it easy to add more appliances to it and combine it with smart home systems for smarter, more sustainable homes.

Overall, this project adds significantly to the emerging area of AI energy efficiency, providing tangible value for consumers, utilities, and the environment.

- Conclusions and Future Recommendation

# Conclusion

In this Final Year Project, we successfully developed and implemented a Non-Intrusive Load Monitoring (NILM) system using an AI-based algorithm to identify and monitor the power consumption of four common household appliances. By focusing on a limited number of appliances, we were able to effectively train and test our algorithm, demonstrating the feasibility and practicality of NILM in a simplified yet realistic home environment.

The system was able to analyze aggregated energy data and accurately disaggregate the power usage of individual devices, providing valuable insights into energy consumption patterns. This not only validates the potential of NILM technology for energy monitoring and conservation but also lays a foundation for future expansion toward a more comprehensive smart home energy management system.

While our project focused on only four appliances due to scope and resource constraints, the framework developed is scalable and adaptable. With further refinement and training on larger datasets, the AI algorithm can be extended to accommodate a wider range of devices, enhancing its application in real-world settings.

Overall, this project highlights the role of artificial intelligence in advancing energy efficiency and promotes the adoption of smart energy solutions in residential sectors.

# Future Recommendation

Future work should include a broader range of household appliances with varying power consumption patterns. This will enhance the system’s scalability and improve its applicability to real-world home environments.

Collecting a larger and more diverse dataset, including data from different households, will help improve the accuracy and generalization of the AI model. Incorporating transfer learning or ensemble methods could also enhance performance.

Deploy the NILM system on a real-time embedded platform such as an STM32 microcontroller or Raspberry Pi with optimized AI inference. This will allow the system to operate autonomously and efficiently in a live household setting.

Bibliography

| [1] | G. Hart, "Prototype Nonintrusive Appliance Load Monitor," Electric Power Research Institute, Vrginia, 1985. |
| --- | --- |
| [2] | M. Dong, P. C. Meira, W. Xu and C. Y. Chung, "Non-intrusive signature extraction for major residential loads," IEEE Transactions on Smart Grid, vol. 4, no. 3, pp. 1421-1430, 2013. |
| [3] | G. Bucci, F. Ciancetta, E. Fiorucci and S. Mari, "Load Identification System for Residential Applications Based on the NILM Technique," in 2020 IEEE International Instrumentation and Measurement Technology Conference (I2MTC), Dubrovnik, 2020. |
| [4] | D. F. Teshome, T. D. Huang and K. L. Lian, "Distinctive load feature extraction basedon fryzes time-domain power theory," IEEE Power and Energy Technology Systems Journal, vol. 3, no. 2, pp. 60-70, 2016. |
| [5] | S. Gupta, M. S. Reynolds and S. N. Patel, "Electrisense: single-point sensing using emi for electrical event detection and classification in the home," in Proceedings of the 12th ACM International Conference on Ubiquitous Computing, New York, ACM, 2010, pp. 139-14. |
| [6] | G. Bucci, F. Ciancetta, E. Fiorucci, S. Mari and A. Fioravanti, "Measurements for non- intrusive load monitoring through machine learning approaches," Acta IMEKO, no. 10, pp. 90-96, 2021. |
| [7] | S. Dash and N. C. Sahoo, "Electric energy disaggregation via non-intrusive load monitoring: A state-of-the-art systematic review," Electric Power Systems Research, vol. 213, 2022. |
| [8] | J. Z. Kolter, S. Batra and A. Y.-T. Ng, "Energy disaggregation via discriminative sparse coding," in 23rd International Conference on Neural Information Processing Systems, Vancouver, 2010. |
| [9] | H. D. Kim, M. Marwah, M. Arlitt, G. Lyon and J. Han, "Unsupervised disaggregation of low frequency power measurements," in Proceedings of the 2011 SIAM International Conference on Data Mining, New York, 2011. |
| [10] | O. Parson, S. Ghosh, M. Weal and A. Rogers, "Non-intrusive load monitoring using prior models of general appliance types," in National Conference on Artificial Intelligence, Toronto, 2012. |
| [11] | R. Bonfigli, E. Principi, . M. Fagiani, M. Severini, S. Squartini and F. Piazza, "Non- intrusive load monitoring by using active and reactive power in," Applied Energy., vol. 208, pp. 1590-1607, 2017. |
| [12] | F. Paradiso, F. Paganelli, D. Giuli and S. Capobianco, "Context-based energy disaggregation in smart homes," Future Internet, vol. 8, no. 1, 2016. |
| [13] | J. Kelly and W. Knottenbelt, "Neural NILM: Deep Neural Networks Applied to Energy Disaggregation," in 2nd ACM International Conference on Embedded Systems for Energy-Efficient Built Environments (BuildSys '15), is the Association for Computing Machinery (ACM)., New York, 2015. |

| [14] | Sense, "Sense Energy Monitor," Sense, January 2022. [Online]. Available: https://sense.com/. |
| --- | --- |
| [15] | S. Mari, . G. Bucci, . F. Ciancetta, . E. Fiorucci and A. Fioravanti, "An Embedded Deep Learning NILM System," in IEEE Transactions on Instrumentation and Measurement, L’Aquila, 2023. |

Appendix

Appendix A:

// Libraries for SD Card Module #include "FS.h"

#include "SD.h" #include "SPI.h"

// SPI Port PINS for SD Card #define REASSIGN_PINS int sck = 14;

int miso = 12; int mosi = 13; int cs = 27;

// Libraries for RTC DS3231 #include <Wire.h>

#include <RTClib.h>

RTC_DS3231 rtc; // Create an instance of the RTC_DS3231 class

// Variables to store received data of iron String I_cycle = "0";

float I_voltage = 0.0; float I_current = 0.0; float I_app_power = 0.0;

float I_react_power = 0.0; float I_act_power = 0.0;

// Variables to store received data of vacuum cleaner String V_cycle = "0";

float V_voltage = 0.0; float V_current = 0.0; float V_app_power = 0.0;

float V_react_power = 0.0; float V_act_power = 0.0;

// Variables to store received data of fridge String F_cycle = "0";

float F_voltage = 0.0; float F_current = 0.0; float F_app_power = 0.0;

float F_react_power = 0.0; float F_act_power = 0.0;

// Variables to store received data of Oven bool O_cycle = 0;

float O_voltage = 0.0; float O_current = 0.0; float O_app_power = 0.0;

float O_react_power = 0.0;

float O_act_power = 0.0; float O_power_fact = 0.0;

// Variables to store received data of Overall Load bool L_cycle = 0;

float L_voltage = 0.0; float L_current = 0.0; float L_app_power = 0.0;

float L_react_power = 0.0; float L_act_power = 0.0; float L_power_fact = 0.0;

String timedate = "NULL"; float voltage = 0.0;

float total_power = 0.0;

//Firebase Setup #include <WiFi.h>

#include <Firebase_ESP_Client.h>

// Your WiFi credentials #define WIFI_SSID "MAUG"

#define WIFI_PASSWORD "d24e95dc"

// Firebase project credentials

#define API_KEY "AIzaSyDBcxIo-G9o1Q_sEJx2Ly1NYT52YibvUFk"

#define DATABASE_URL "https://fyp-aiot-smart-meter-dataset-default-rtdb.asia- southeast1.firebasedatabase.app/" // Database URL

FirebaseData fbdo; FirebaseConfig config; FirebaseAuth auth;

unsigned long lastTime = 0;

unsigned long timerDelay = 5000; // Send data every 5 second

void setup() {

// Initialize Serial ports

Serial.begin(9600);	// Oven (Appliance 04)

Serial1.begin(9600, SERIAL_8N1, 16, 17);		// Total (Overall Power Consumption) Serial2.begin(9600, SERIAL_8N1, 4, 2);	// For serial Communication with other ESP32 for other Appliances Values

Serial.println("Waiting for data...");

// Initialize RTC Date & Time Wire.begin();

if (!rtc.begin()) { Serial.println("Couldn't find RTC"); while (1);

}

// Uncomment this if you want to reset the time and date

// Set RTC to the current Unix time

// unsigned long unixTime = 1733249000; // Example Unix timestamp

// rtc.adjust(DateTime(unixTime));

// Initialize SD Card #ifdef REASSIGN_PINS

SPI.begin(sck, miso, mosi, cs); if (!SD.begin(cs))

{

#else

if (!SD.begin())

{

#endif

Serial.println("Card Mount Failed"); delay(100);

}

uint8_t cardType = SD.cardType(); while (cardType == CARD_NONE)

{

Serial.println("No SD card attached"); delay(100);

}

WiFi.begin(WIFI_SSID, WIFI_PASSWORD);

while (WiFi.status() != WL_CONNECTED) { delay(1000);

Serial.println("Connecting to Wi-Fi...");

}

Serial.println("Connected to Wi-Fi");

// Firebase configuration config.api_key = API_KEY;

config.database_url = DATABASE_URL;

// Assign the user authentication (optional)

auth.user.email = "ghauribrother12@gmail.com";	// Optional for authentication auth.user.password = "MAhm12##"; // Optional for authentication

// Initialize Firebase Firebase.begin(&config, &auth); Firebase.reconnectWiFi(true);

}

void loop() {

// Check if data is available on Serial (IRON) if (Serial.available() > 0) {

String receivedData = Serial.readStringUntil('\n'); // Read the data line

// Calling Function to calculate values of iron ovenData(receivedData);

}

// Check if data is available on Serial1 (VACUUM CLEANER) if (Serial1.available() > 0) {

String receivedData_1 = Serial1.readStringUntil('\n'); // Read the data line

// Calling Function to calculate values of vacuum cleaner

loadData(receivedData_1);

}

// Check if data is available on Serial2 (FRIDGE) if (Serial2.available() > 0) {

String receivedData_2 = Serial2.readStringUntil('\n'); // Read the data line

// Calling Function to calculate values of fridge otherData(receivedData_2);

}

// Format the date and time as a string DateTime now = rtc.now();

uint32_t unixTime = now.unixtime();

timedate = String(now.year()) + String(now.month()) + String(now.day()) + " " + String(unixTime);

// Storing Data in SD Card

File file = SD.open("/hello.txt", FILE_APPEND); while (!file)

{

Serial.println("Failed to open file for writing"); delay(100);

}

file.print(timedate); file.print(",");

voltage = (I_voltage+V_voltage+F_voltage+O_voltage)/4; file.print(voltage);

file.print(",");

file.print(I_current); file.print(","); file.print(V_current); file.print(","); file.print(F_current); file.print(","); file.print(O_current); file.print(",");

total_power = I_act_power+V_act_power+F_act_power+O_act_power; file.print(total_power);

file.print(",");

file.print(I_act_power); file.print(","); file.print(V_act_power); file.print(","); file.print(F_act_power); file.print(","); file.print(O_act_power); file.print(",");

file.print(I_react_power); file.print(",");

file.print(V_react_power); file.print(","); file.print(F_react_power); file.print(","); file.print(O_react_power); file.print(",");

file.print(I_app_power); file.print(","); file.print(V_app_power); file.print(","); file.print(F_app_power); file.print(","); file.print(O_app_power); file.print(",");

file.print(I_cycle); file.print(","); file.print(V_cycle); file.print(","); file.print(F_cycle); file.print(","); file.print(O_cycle); file.println();

file.close(); overallData();

// Send data every `timerDelay` milliseconds if ((millis() - lastTime) > timerDelay) { lastTime = millis();

// JSON object to store data

String path = "/sensorData/" + String(millis()); // Timestamp as key FirebaseJson json;

// Populate JSON object with the data json.set("/TimeDate",timedate);

json.set("/Voltage",voltage);

json.set("/Iron_Current",I_current); json.set("/Vacuum_Current",V_current); json.set("/Fridge_Current",F_current); json.set("/Oven_Current",O_current);

json.set("/Total_Power",total_power);

json.set("/Iron_Act_Power",I_act_power); json.set("/Vacuum_Act_Power",V_act_power); json.set("/Fridge_Act_Power",F_act_power); json.set("/Oven_Act_Power",O_act_power);

json.set("/Iron_ReAct_Power",I_react_power); json.set("/Vacuum_ReAct_Power",V_react_power);

json.set("/Fridge_ReAct_Power",F_react_power); json.set("/Oven_ReAct_Power",O_react_power);

json.set("/Iron_App_Power",I_app_power); json.set("/Vacuum_App_Power",V_app_power); json.set("/Fridge_App_Power",F_app_power); json.set("/Oven_App_Power",O_app_power);

json.set("/Iron_Cycle",I_cycle); json.set("/Vacuum_Cycle",V_cycle); json.set("/Fridge_Cycle",F_cycle); json.set("/Oven_Cycle",O_cycle);

// Send data to Firebase

if (Firebase.RTDB.setJSON(&fbdo, path.c_str(), &json)) { Serial.println("Data sent successfully:");

} else {

Serial.print("Failed to send data: "); Serial.println(fbdo.errorReason());

}

}

delay(2000); // Delay to avoid flooding with data

}

void overallData() {

File file = SD.open("/alldata.txt", FILE_APPEND); while (!file)

{

Serial.println("Failed to open file for writing"); delay(100);

}

file.print(timedate); file.print(","); file.print(L_voltage); file.print(","); file.print(L_current); file.print(","); file.print(L_act_power); file.print(","); file.print(L_react_power); file.print(","); file.print(L_app_power); file.print(","); file.print(L_power_fact); file.print(","); file.print(I_cycle); file.print(","); file.print(V_cycle); file.print(","); file.print(F_cycle); file.print(","); file.print(O_cycle);

file.println(); // Add a newline at the end

file.close();

}

// Function to Oven data void ovenData(String data) {

int ovoltageIndex = data.indexOf("Voltage:"); int ocurrentIndex = data.indexOf("Current:");

int oapp_powerIndex = data.indexOf("Apparent Power:"); int opower_factIndex = data.indexOf("Power Factor:");

if (ovoltageIndex != -1 && ocurrentIndex != -1 && oapp_powerIndex != -1 && opower_factIndex != -1) {

// Extract the voltage value

O_voltage = data.substring(ovoltageIndex + 8, data.indexOf("V", ovoltageIndex)).toFloat();

// Extract the current value

O_current = data.substring(ocurrentIndex + 8, data.indexOf("A", ocurrentIndex)).toFloat();

// Extract the apparent power value

O_app_power = data.substring(oapp_powerIndex + 15, data.indexOf("VA", oapp_powerIndex)).toFloat();

// Extract the power factor value

O_power_fact = data.substring(opower_factIndex + 13, data.indexOf("\n", opower_factIndex)).toFloat();

// Calculation of State of Appliance if (O_current >= 0.5){

O_cycle = 1;

}

// Calculation of active power

O_act_power = O_app_power * O_power_fact;

// Calculation of reactive power

O_react_power = sqrt(pow(O_app_power,2) - pow(O_act_power,2));

}

}

// Function to Total Load data void loadData(String data) {

int lvoltageIndex = data.indexOf("Voltage:"); int lcurrentIndex = data.indexOf("Current:");

int lapp_powerIndex = data.indexOf("Apparent Power:"); int lpower_factIndex = data.indexOf("Power Factor:");

if (lvoltageIndex != -1 && lcurrentIndex != -1 && lapp_powerIndex != -1 && lpower_factIndex != -1) {

// Extract the voltage value

L_voltage = data.substring(lvoltageIndex + 8, data.indexOf("V", lvoltageIndex)).toFloat();

// Extract the current value

L_current = data.substring(lcurrentIndex + 8, data.indexOf("A", lcurrentIndex)).toFloat();

// Extract the apparent power value

L_app_power = data.substring(lapp_powerIndex + 15, data.indexOf("VA", lapp_powerIndex)).toFloat();

// Extract the power factor value

L_power_fact = data.substring(lpower_factIndex + 13, data.indexOf("\n", lpower_factIndex)).toFloat();

// Calculation of State of Appliance if (L_current >= 0.5){

L_cycle = 1;

}

// Calculation of active power

L_act_power = L_app_power * L_power_fact;

// Calculation of reactive power

L_react_power = sqrt(pow(L_app_power,2) - pow(L_act_power,2));

}

}

// Function to Iron, Vacuum, Fridge data void otherData(String data) {

int ivoltageIndex = data.indexOf("Iron Voltage: "); int icurrentIndex = data.indexOf("Iron Current: ");

int iact_powerIndex = data.indexOf("Iron Active Power: ");

int ireact_powerIndex = data.indexOf("Iron Reactive Power: "); int iapp_powerIndex = data.indexOf("Iron Apparent Power: "); int icycleIndex = data.indexOf("Iron Cycle: ");

if (ivoltageIndex != -1 && icurrentIndex != -1 && iact_powerIndex != -1 && ireact_powerIndex != -1 && iapp_powerIndex != -1 && icycleIndex != -1) {

// Extract the voltage value

I_voltage = data.substring(ivoltageIndex + 14, data.indexOf(" V", ivoltageIndex)).toFloat();

// Extract the current value

I_current = data.substring(icurrentIndex + 14, data.indexOf(" A", icurrentIndex)).toFloat();

// Extract the active power value

I_act_power = data.substring(iact_powerIndex + 19, data.indexOf(" W", iact_powerIndex)).toFloat();

// Extract the reactive power value

I_react_power = data.substring(ireact_powerIndex + 21, data.indexOf(" VAR", ireact_powerIndex)).toFloat();

// Extract the apparent power value

I_app_power = data.substring(iapp_powerIndex + 21, data.indexOf(" VA", iapp_powerIndex)).toFloat();

// Extract the cycle value

I_cycle = data.substring(icycleIndex + 12, data.indexOf("\n", icycleIndex));

}

int vvoltageIndex = data.indexOf("Vacuum Voltage: "); int vcurrentIndex = data.indexOf("Vacuum Current: ");

int vact_powerIndex = data.indexOf("Vacuum Active Power: ");

int vreact_powerIndex = data.indexOf("Vacuum Reactive Power: "); int vapp_powerIndex = data.indexOf("Vacuum Apparent Power: "); int vcycleIndex = data.indexOf("Vacuum Cycle: ");

if (vvoltageIndex != -1 && vcurrentIndex != -1 && vact_powerIndex != -1 && vreact_powerIndex != -1 && vapp_powerIndex != -1 && vcycleIndex != -1) {

// Extract the voltage value

V_voltage = data.substring(vvoltageIndex + 16, data.indexOf(" V", vvoltageIndex)).toFloat();

// Extract the current value

V_current = data.substring(vcurrentIndex + 16, data.indexOf(" A", vcurrentIndex)).toFloat();

// Extract the active power value

V_act_power = data.substring(vact_powerIndex + 21, data.indexOf(" W", vact_powerIndex)).toFloat();

// Extract the reactive power value

V_react_power = data.substring(vreact_powerIndex + 23, data.indexOf(" VAR", vreact_powerIndex)).toFloat();

// Extract the apparent power value

V_app_power = data.substring(vapp_powerIndex + 23, data.indexOf(" VA", vapp_powerIndex)).toFloat();

// Extract the cycle value

V_cycle = data.substring(vcycleIndex + 14, data.indexOf("\n", vcycleIndex));

}

int fvoltageIndex = data.indexOf("Fridge Voltage: "); int fcurrentIndex = data.indexOf("Fridge Current: ");

int fact_powerIndex = data.indexOf("Fridge Active Power: ");

int freact_powerIndex = data.indexOf("Fridge Reactive Power: "); int fapp_powerIndex = data.indexOf("Fridge Apparent Power: "); int fcycleIndex = data.indexOf("Fridge Cycle: ");

if (fvoltageIndex != -1 && fcurrentIndex != -1 && fact_powerIndex != -1 && freact_powerIndex != -1 && fapp_powerIndex != -1 && fcycleIndex != -1) {

// Extract the voltage value

F_voltage = data.substring(fvoltageIndex + 16, data.indexOf(" V", fvoltageIndex)).toFloat();

// Extract the current value

F_current = data.substring(fcurrentIndex + 16, data.indexOf(" A", fcurrentIndex)).toFloat();

// Extract the active power value

F_act_power = data.substring(fact_powerIndex + 21, data.indexOf(" W", fact_powerIndex)).toFloat();

// Extract the reactive power value

F_react_power = data.substring(freact_powerIndex + 23, data.indexOf(" VAR", freact_powerIndex)).toFloat();

// Extract the apparent power value

F_app_power = data.substring(fapp_powerIndex + 23, data.indexOf(" VA", fapp_powerIndex)).toFloat();

// Extract the cycle value

F_cycle = data.substring(fcycleIndex + 14, data.indexOf("\n", fcycleIndex));

}

}

Appendix B:

// Libraries for SD Card Module #include "FS.h"

#include "SD.h" #include "SPI.h"

// SPI Port PINS for SD Card #define REASSIGN_PINS int sck = 14;

int miso = 12; int mosi = 13; int cs = 27;

// Libraries for RTC DS3231 #include <Wire.h>

#include <RTClib.h>

RTC_DS3231 rtc; // Create an instance of the RTC_DS3231 class

// Variables to store received data of iron bool I_cycle = 0;

float I_voltage = 0.0; float I_current = 0.0; float I_app_power = 0.0;

float I_react_power = 0.0; float I_act_power = 0.0; float I_power_fact = 0.0;

// Variables to store received data of vacuum cleaner bool V_cycle = 0;

float V_voltage = 0.0; float V_current = 0.0; float V_app_power = 0.0;

float V_react_power = 0.0; float V_act_power = 0.0; float V_power_fact = 0.0;

// Variables to store received data of fridge bool F_cycle = 0;

float F_voltage = 0.0; float F_current = 0.0; float F_app_power = 0.0;

float F_react_power = 0.0; float F_act_power = 0.0; float F_power_fact = 0.0;

float voltage = 0.0; float total_power = 0.0;

String dateTimeStr = "NULL";

void setup() {

// Initialize Serial ports

Serial.begin(9600);

Serial1.begin(9600, SERIAL_8N1, 16, 17);

Serial2.begin(9600, SERIAL_8N1, 4, 2); Serial.println("Waiting for data...");

// Initialize RTC Date & Time Wire.begin();

if (!rtc.begin()) { Serial.println("Couldn't find RTC"); while (1);

}

// Uncomment this if you want to reset the time and date

// Set RTC to the current Unix time

// unsigned long unixTime = 1733248700; // Example Unix timestamp

// rtc.adjust(DateTime(unixTime));

// Initialize SD Card #ifdef REASSIGN_PINS

SPI.begin(sck, miso, mosi, cs); if (!SD.begin(cs))

{

#else

if (!SD.begin())

{

#endif

Serial.println("Card Mount Failed"); delay(100);

}

uint8_t cardType = SD.cardType(); while (cardType == CARD_NONE)

{

Serial.println("No SD card attached"); delay(100);

}

}

void loop() {

// Check if data is available on Serial (IRON) if (Serial.available() > 0) {

String receivedData = Serial.readStringUntil('\n'); // Read the data line

// Calling Function to calculate values of iron ironData(receivedData);

}

// Check if data is available on Serial1 (VACUUM CLEANER) if (Serial1.available() > 0) {

String receivedData_1 = Serial1.readStringUntil('\n'); // Read the data line

// Calling Function to calculate values of vacuum cleaner vacuumData(receivedData_1);

}

// Check if data is available on Serial2 (FRIDGE) if (Serial2.available() > 0) {

String receivedData_2 = Serial2.readStringUntil('\n'); // Read the data line

// Calling Function to calculate values of fridge fridgeData(receivedData_2);

}

// Format the date and time as a string DateTime now = rtc.now();

uint32_t unixTime = now.unixtime();

dateTimeStr = String(now.year()) + String(now.month()) + String(now.day()) + " " + String(unixTime);

// Storing Data in SD Card

File file = SD.open("/hello.txt", FILE_APPEND); while (!file)

{

Serial.println("Failed to open file for writing"); delay(100);

}

file.print(dateTimeStr); file.print(",");

voltage = (I_voltage+V_voltage+F_voltage)/3; file.print(voltage);

file.print(",");

file.print(I_current); file.print(","); file.print(V_current); file.print(","); file.print(F_current); file.print(",");

total_power = I_act_power+V_act_power+F_act_power; file.print(total_power);

file.print(",");

file.print(I_act_power); file.print(","); file.print(V_act_power); file.print(","); file.print(F_act_power); file.print(",");

file.print(I_react_power); file.print(","); file.print(V_react_power); file.print(","); file.print(F_react_power); file.print(",");

file.print(I_app_power); file.print(","); file.print(V_app_power);

file.print(","); file.print(F_app_power); file.print(",");

file.print(I_cycle); file.print(","); file.print(V_cycle); file.print(","); file.print(F_cycle); file.println();

file.close();

// Send Value to the other ESP32 using UART Communication Serial.println("	");

Serial.println("Iron Voltage: " + String(I_voltage) + " V"); Serial.println("Iron Current: " + String(I_current) + " A"); Serial.println("Iron Active Power: " + String(I_act_power) + " W"); Serial.println("Iron Reactive Power: " + String(I_react_power) + " VAR"); Serial.println("Iron Apparent Power: " + String(I_app_power) + " VA"); Serial.println("Iron Cycle: " + String(I_cycle));

Serial.println("	");

Serial.println("Vacuum Voltage: " + String(V_voltage) + " V"); Serial.println("Vacuum Current: " + String(V_current) + " A"); Serial.println("Vacuum Active Power: " + String(V_act_power) + " W"); Serial.println("Vacuum Reactive Power: " + String(V_react_power) + " VAR"); Serial.println("Vacuum Apparent Power: " + String(V_app_power) + " VA"); Serial.println("Vacuum Cycle: " + String(V_cycle));

Serial.println("	");

Serial.println("Fridge Voltage: " + String(F_voltage) + " V"); Serial.println("Fridge Current: " + String(F_current) + " A"); Serial.println("Fridge Active Power: " + String(F_act_power) + " W"); Serial.println("Fridge Reactive Power: " + String(F_react_power) + " VAR"); Serial.println("Fridge Apparent Power: " + String(F_app_power) + " VA"); Serial.println("Fridge Cycle: " + String(F_cycle));

delay(2000); // Delay to avoid flooding with data

}

// Function to Iron data

void ironData(String data) {

int ivoltageIndex = data.indexOf("Voltage:"); int icurrentIndex = data.indexOf("Current:");

int iapp_powerIndex = data.indexOf("Apparent Power:"); int ipower_factIndex = data.indexOf("Power Factor:");

if (ivoltageIndex != -1 && icurrentIndex != -1 && iapp_powerIndex != -1 && ipower_factIndex != -1) {

// Extract the voltage value

I_voltage = data.substring(ivoltageIndex + 8, data.indexOf("V", ivoltageIndex)).toFloat();

// Extract the current value

I_current = data.substring(icurrentIndex + 8, data.indexOf("A", icurrentIndex)).toFloat();

// Extract the apparent power value

I_app_power = data.substring(iapp_powerIndex + 15, data.indexOf("VA", iapp_powerIndex)).toFloat();

// Extract the power factor value

I_power_fact = data.substring(ipower_factIndex + 13, data.indexOf("\n", ipower_factIndex)).toFloat();

// Calculation of State of Appliance if (I_current >= 0.5){

I_cycle = 1;

}

// Calculation of active power

I_act_power = I_app_power * I_power_fact;

// Calculation of reactive power

I_react_power = sqrt(pow(I_app_power,2) - pow(I_act_power,2));

}

}

// Function to Vacuum data void vacuumData(String data) {

int vvoltageIndex = data.indexOf("Voltage:"); int vcurrentIndex = data.indexOf("Current:");

int vapp_powerIndex = data.indexOf("Apparent Power:"); int vpower_factIndex = data.indexOf("Power Factor:");

if (vvoltageIndex != -1 && vcurrentIndex != -1 && vapp_powerIndex != -1 && vpower_factIndex != -1) {

// Extract the voltage value

V_voltage = data.substring(vvoltageIndex + 8, data.indexOf("V", vvoltageIndex)).toFloat();

// Extract the current value

V_current = data.substring(vcurrentIndex + 8, data.indexOf("A", vcurrentIndex)).toFloat();

// Extract the apparent power value

V_app_power = data.substring(vapp_powerIndex + 15, data.indexOf("VA", vapp_powerIndex)).toFloat();

// Extract the power factor value

V_power_fact = data.substring(vpower_factIndex + 13, data.indexOf("\n", vpower_factIndex)).toFloat();

// Calculation of State of Appliance if (V_current >= 0.5){

V_cycle = 1;

}

// Calculation of active power

V_act_power = V_app_power * V_power_fact;

// Calculation of reactive power

V_react_power = sqrt(pow(V_app_power,2) - pow(V_act_power,2));

}

}

// Function to Fridge data void fridgeData(String data) {

// Example: "Voltage: 230.0 V Current: 10.00 A Power: 2300.00 W"

int fvoltageIndex = data.indexOf("Voltage:"); int fcurrentIndex = data.indexOf("Current:");

int fapp_powerIndex = data.indexOf("Apparent Power:"); int fpower_factIndex = data.indexOf("Power Factor:");

if (fvoltageIndex != -1 && fcurrentIndex != -1 && fapp_powerIndex != -1 && fpower_factIndex != -1) {

// Extract the voltage value

F_voltage = data.substring(fvoltageIndex + 8, data.indexOf("V", fvoltageIndex)).toFloat();

// Extract the current value

F_current = data.substring(fcurrentIndex + 8, data.indexOf("A", fcurrentIndex)).toFloat();

// Extract the apparent power value

F_app_power = data.substring(fapp_powerIndex + 15, data.indexOf("VA", fapp_powerIndex)).toFloat();

// Extract the power factor value

F_power_fact = data.substring(fpower_factIndex + 13, data.indexOf("\n", fpower_factIndex)).toFloat();

// Calculation of State of Appliance if (F_current >= 0.5){

F_cycle = 1;

}

// Calculation of active power

F_act_power = F_app_power * F_power_fact;

// Calculation of reactive power

F_react_power = sqrt(pow(F_app_power,2) - pow(F_act_power,2));

}

}